## 1. Physical Interface & Communication Settings

The system uses a standard CAN 2.0B interface with the following parameters:

- Baud Rate: 500K (Default, user-definable).

- Identifier (ID): 11-bit standard ID.

- Mechanism: Master-Slave / Query-Response.

  - The BMS only responds after receiving an instruction from the host.

  - Host Query: The host sends a data frame with a specific ID. It is recommended to include one byte of data (0x5A).

  - BMS Response: The BMS responds with the same ID, a Data Length Code (DLC) of 8, and the requested data.

- Byte Order: All multi-byte data is transmitted Big-Endian (High byte first, Low byte last).

## 2. Real-Time Data Monitoring (IDs 0x100 - 0x110)

These identifiers are used to query the current status of the battery pack. Each response consists of 6 bytes of data followed by a 2-byte CRC-16 checksum.

| CAN ID | Content | Data Length | Byte Description |
| --- | --- | --- | --- |
| 0x100 | Basic Status | 6 + 2 Checksum | B0-1: Total Voltage (Unit: 10mV, Unsigned) B2-3: Current (Unit: 10mA, Signed, Positive=Charge) B4-5: Remaining Capacity (Unit: 10mAh, Unsigned) |
| 0x101 | Capacity & Cycles | 6 + 2 Checksum | B0-1: Full Capacity (Unit: 10mAh) B2-3: Cycle Count (Unit: 1 time) B4-5: RSOC (Unit: %) |
| 0x102 | Balancing & Protection | 6 + 2 Checksum | B0-1: Balance Status (B0: Cell 1-8, B1: Cell 9-16) B2-3: Balance Status (B2: Cell 17-24, B3: Cell 25-33) B4-5: Protection Status (See Note 1) |
| 0x103 | FET & Version | 6 + 2 Checksum | B0-1: MOSFET Status (See Note 2) +1B2-3: Production Date (See Note 3) +1B4-5: Software Version |
| 0x104 | Hardware Specs | 2 + 2 Checksum | B0: Number of battery strings B1: Number of NTC temperature probes |
| 0x105 | Temperatures 1-3 | 6 + 2 Checksum | B0-1: NTC1 / B2-3: NTC2 / B4-5: NTC3 (Unit: 0.1K, See Note 4) +1 |
| 0x107 | Cell Voltages 1-3 | 6 + 2 Checksum | B0-1: Cell 1 / B2-3: Cell 2 / B4-5: Cell 3 (Unit: 1mV) |
| ... | ... | ... | Incremental IDs (0x108-0x110) follow the pattern for subsequent cells. |

## 3. Parameter Configuration & Control

These IDs allow for manual control of the BMS or reading/writing protection parameters.

| CAN ID | Function | Length | Description |
| --- | --- | --- | --- |
| 0x12D | MOSFET Control | 4 Bytes | B0: Discharge MOS Control / B1: Inverse of B0 B2: Charge MOS Control / B3: Inverse of B2 (0 = Open, 1 = Close. Must use command to reopen) |
| 0x12E | Factory Mode | 1 Byte | Data: 0x5678 to enter |
| 0x12F | Init EEPROM | 2 Bytes | Data: 0x2828 to write parameters to EEPROM |
| 0x130 | Protect Params 1-4 | 1 or 8 Bytes | Length 1 (Data 0x5A) = Read. Length 8 = Write. Includes Design Cap, Cycle Cap, Full Charge V, Cut-off V. |
| 0x131 | Protect Params 5-8 | 8 Bytes | Self-discharge rate, Date, Serial Number, Cycle Count |
| 0x137 | Config Params | 8 Bytes | Sample resistor, Function config, NTC config, String count |

## 4. Critical Technical Notes

### Note 1: Protection Status Flags (ID 0x102, B4-5)

A bit value of 1 indicates the protection is active.

| Bit | Meaning | Bit | Meaning |
| --- | --- | --- | --- |
| bit0 | Cell Overvoltage | bit6 | Discharge Over-temp |
| bit1 | Cell Undervoltage | bit7 | Discharge Under-temp |
| bit2 | Pack Overvoltage | bit8 | Charge Overcurrent |
| bit3 | Pack Undervoltage | bit9 | Discharge Overcurrent |
| bit4 | Charge Over-temp | bit10 | Short Circuit Protection |
| bit5 | Charge Under-temp | bit11 | IC Error (Front-end) |

This document outlines the CANBUS communication protocol for JBD Battery Management Systems (BMS). It covers the physical interface requirements, communication mechanism, data frame structures, and checksum calculations.