## 1. Physical Interface & Communication Settings

- Physical Port: Standard CAN interface.

- Baud Rate: The default baud rate is 500K, but it is user-definable.

- Identifier Format: Uses an 11-bit standard identifier (ID) within a CAN 2.0B frame.

- Communication Mechanism: The system utilizes a query-response mechanism where the BMS only responds after receiving a command from the host.

- Query Format: The host sends a specific ID and data frame; it is recommended to include one byte of data (typically 0x5A) to trigger a response.

- Response Format: The BMS responds with a DLC of 8, returning 8 bytes of data.

- Byte Order: All multi-byte data is transmitted Big-Endian (High byte first, Low byte last).

- Checksum: Data integrity is verified using CRC-16.

## 2. Communication Protocol Identifiers

The following table details the identifiers used for querying battery data and protection status:

| ID | Description | Length | Details |
| --- | --- | --- | --- |
| 0x100 | Basic Pack Info | 6+2 Checksum | B0-1: Total Voltage (10mV unit, Unsigned)B2-3: Current (10mA unit, Signed; Charge is +, Discharge is -)B4-5: Remaining Capacity (10mAh unit) |
| 0x101 | Capacity & Cycle Info | 6+2 Checksum | B0-1: Full Capacity (10mAh unit)B2-3: Discharge CyclesB4-5: RSOC (Relative State of Charge, %) |
| 0x102 | Balancing & Protection | 6+2 Checksum | B0-1: Balance Status (Cell 1–16: 1=On, 0=Off)B2-3: Balance Status (Cell 17–33: 1=On, 0=Off)B4-5: Protection Status Flags (See Note 1) |
| 0x103 | FET & Hardware Info | 6+2 Checksum | B0-1: MOSFET Status (See Note 2)B2-3: Production Date (See Note 3)B4-5: Software Version |
| 0x104 | Hardware Configuration | 2+2 Checksum | B0: Number of Cell StringsB1: Number of NTC Temperature Probes |
| 0x105 | Temperature Data 1 | 6+2 Checksum | B0-1: NTC1 TempB2-3: NTC2 TempB4-5: NTC3 Temp (Units in 0.1K, see Note 4) |
| 0x107 | Cell Voltage Group 1 | 6+2 Checksum | B0-1: Cell 1 Voltage (mV)B2-3: Cell 2 Voltage (mV)B4-5: Cell 3 Voltage (mV) |
| 0x108 - 0x110 | Cell Voltage Groups | 6+2 Checksum | Follows the same pattern for subsequent cell groups (Cell 4 to Cell 30). |
| 0x111 - 0x114 | Device Model | 8 Bytes | Hardware device model details. |

2.1 Configuration and Control IDs

| ID | Function | Length | Details |
| --- | --- | --- | --- |
| 0x12D | MOSFET Switch Control | 4 Bytes | B0: Discharge MOS (0=Open, 1=Close)B1: Inverse of B0B2: Charge MOS (0=Open, 1=Close)B3: Inverse of B2 |
| 0x12E | Factory Mode | 1 Byte | Data 0x5678 to enter factory mode. |
| 0x12F | EEPROM Initialization | - | Data 0x2828 writes parameters to EEPROM. |
| 0x130 - 0x14A | Protection Parameters | 8 Bytes | Parameters including Overvoltage, Undervoltage, Temperature, and Current limits. If data length is 1 and data is 0x5A, it performs a Read; otherwise, it is a Write. |

## 3. Special Explanations

### Note 1: Protection Status Flag Definition (ID 0x102, Bytes 4–5)

A bit value of 1 indicates the protection is active13:

- Bit 0: Cell Overvoltage Protection 14

- Bit 1: Cell Undervoltage Protection 15

- Bit 2: Pack Overvoltage Protection 16

- Bit 3: Pack Undervoltage Protection 17

- Bit 4: Charge Over-temperature Protection 18

- Bit 5: Charge Under-temperature Protection 19

- Bit 6: Discharge Over-temperature Protection 20

- Bit 7: Discharge Under-temperature Protection 21

- Bit 8: Charge Over-current Protection 22

- Bit 9: Discharge Over-current Protection 23

- Bit 10: Short Circuit Protection 24

- Bit 11: Front-end IC Error 25

- Bit 12: Software Lock MOSFET 26

### Note 2: MOSFET Status (ID 0x103, Byte 0–1)

- Bit 0: Charging MOS (0 = Off, 1 = On)27.

- Bit 1: Discharging MOS (0 = Off, 1 = On)28.

### Note 3: Production Date Resolution

The date is encoded in 2 bytes29.

- Day: Value & 0x1F.

- Month: (Value >> 5) + 0x0F.

- Year: 2000 + (Value >> 9).

- Example: 0x2068 results in March 8, 2016.

### Note 4: Temperature Resolution

Temperatures are transmitted in absolute temperature (Kelvin) scaled by 1034:

Transmitted Value=2731+Actual Celsius×10

- 0∘C=2731

- 25∘C=2981

- -10∘C=2631

## 4. CRC-16 Calculation

The protocol uses a CRC-16 polynomial 0xA001.

Implementation (C Code)

#define CRC_16_POLYNOMIALS 0xa001

uint16_t Check_CRC16(uint8_t* pchMsg, uint8_t wDataLen) {
    uint8_t i, chChar;
    uint16_t wCRC = 0xFFFF;
    while (wDataLen--) {
        chChar = *pchMsg++;
        wCRC ^= (uint16_t)chChar;
        for (i = 0; i < 8; i++) {
            if (wCRC & 0x0001)
                wCRC = (wCRC >> 1) ^ CRC_16_POLYNOMIALS;
            else
                wCRC >>= 1;
        }
    }
    return wCRC;
}

The Host Query

To request the basic pack information, which includes the total voltage, you send a CAN frame with the identifier 0x100.

CAN ID: 0x100

DLC (Data Length Code): 1

Data Payload: 0x5A (Recommended trigger byte)

2. The BMS Response

The BMS will respond with an 8-byte data frame (DLC = 8) containing the raw values and a CRC-16 checksum.

Example Hex Response: 13 88 00 FB 09 C4 73 1B

| Byte Index | Hex Value | Description |
| --- | --- | --- |
| BYTE 0~1 | 13 88 | Total Voltage: Unsigned integer, high byte first |
| BYTE 2~3 | 00 FB | Current: Signed integer |
| BYTE 4~5 | 09 C4 | Remaining Capacity: Unsigned integer |
| BYTE 6~7 | 73 1B | CRC-16 Checksum: Verification for Bytes 0-5 |

### 3. Calculating the Human-Readable Value

According to the manual, the total voltage is provided in units of 10mV.

- Convert Hex to Decimal:

The hex value 0x1388 converts to 5000 in decimal.

- Apply the Unit Scaling:

5000 * 10mV = 50,000mV

- Convert to Volts:

50,000mV / 1000 = 50.00V